Supplementary Material

Methods and Materials

Kynurenine, tryptophan, and serotonin quantification in plasma

Plasma levels of kynurenine (KYN), tryptophan (TRP), and serotonin (5-HT) were measured using High-performance liquid chromatography (HPLC). Plasma aliquots (100 μl) were deproteinized by adding 25 μl of 6% perchloric acid, and tyrosine (25 μl of 50 μM) was added as internal standard, to a final dilution of 1:5. The acidified samples were vortexed, incubated at room temperature (10 min), and centrifuged (15 min, 16,000×g, at 4 C) to collect supernatants. After centrifugation, plasma supernatants were purified using 0.2 μm filters (Minisart® RC 4, Sartorius, Thermo Fisher Scientific, MA, USA). For TRP and 5-HT measurement, 20 μl of the supernatant was applied to a reversed-phase column (Spherisorb ODS2; 5 μm; 150 × 4.6 mm; Thermo Scientific, Massachusetts, USA), and TRP and 5-HT were isocratically eluted using a mobile phase (0.1 M ammonium acetate and 8% methanol, pH 3.8), at a flow rate of 1 ml/min. TRP and 5-HT were detected fluorometrically at excitation/emission wavelengths of 270/360 nm and 290/398 nm, respectively (Waters 2475, Multi fluorescence Detector; Waters, Milford, MA, USA). For KYN measurement, 20 μl of the supernatant was applied to the same column as above but separated using a mobile phase containing 0.1 M sodium acetate and 4% acetonitrile, pH 4.6, delivered at 1 ml/min. KYN was measured by UV detection (365 nm, Waters 2487). Retention times were approximately 7, 4.7, and 5 min for TRP, 5-HT, and KYN respectively. The ratios of KYN or 5-HT to TRP concentrations were calculated and used as a measure of TRP degradation.

Detailed library preparation and sequencing methodology

Two serial dilutions of DNA samples were used for quality control purposes. Genomic DNA was quantified using SYBRGreen I (Sigma-Aldrich, Merck), and the total bacterial load was estimated for each sample by real-time PCR assay (47) (7900 HT Fast Real-Time PCR System, Applied Biosystems) for normalization of bacterial load before library preparation. The 16S rRNA V3-V4 region was amplified using universal primers in a limited cycle PCR: V3-V4-Forward (5′-TCGTCGGCAGCGTCAGATGTGTATAAGAGACAGCCTACGGGNGGCWGCAG-3′) V3-V4-Reverse (5′-GTCTCGTGGGCTCGGAGATGTGTATAAGAGACAGGACTACHVGGGTATCTAATCC-3′), as described by Willis and colleagues (48), with slight modifications. To prevent unbalanced base composition in the final sequencing phase, various numbers of "N" bases (between 1 and 4) were added as spacers to both forward and reverse primers (a total of 4 forward and 4 reverse primers were used). First, normalized DNA samples were used to amplify the V3-V4 regions of 16S ribosomal RNA gene, in a limited cycle PCR. The PCR was performed in 25 µl volume with 0.08 µM primer concentration and NEBNext Q5 Hot Start HiFi PCR Master Mix (M0543L, New England Biolabs), with cycling conditions: initial denaturation of 30 s at 98 °C, 5 cycles of 98 °C for 10 s, 55 °C for 5 min, and 65 °C for 45 s.

This was followed by a second PCR to add full-length Nextera adapters; the PCR reactions consisted of NEBNext Q5 Hot Start HiFi PCR Master Mix and UDI-Nextera primers (IDT), with the following cycling conditions: denaturation of 30 s at 98 °C, 17 cycles of 98 °C for 10 s, 55 °C for 30 s, and 65 °C for 45 s, and 5 min at 65 °C. Libraries were purified using AgenCourt AMPure XP beads (A63882, Beckman Coulter) with a 0.8X ratio according to the manufacturer's instructions and analyzed using Fragment Analyzer (DNF-915, Agilent Biosystems) to estimate the quantity and determine the size distribution. A pool of normalized libraries, quantified by qPCR using KAPA Library Quantification Kit KK4835 (07960204001, Roche) on a 7900 HT Fast Real-Time PCR System, was prepared for subsequent sequencing. Sequencing was performed on Illumina's NextSeq2000 on a P1 flow cell with 2 x 300 bp reads (loading concentration of 900 pM). To increase the diversity of the sequences, 40% of PhIX control library was added. Negative (NTC as well as a negative extraction control) and positive (ZymoBIOMICS(TM) Microbial Community DNA Standard; D6306, Zymo) controls were included with the samples, and the PCR amplification steps were performed in parallel, using the same conditions and reagents.